2008 Solution

Part A

Step 1 Draw the glass

Single Glazing				Double Glazing

Step 2 Draw data table for Single Glazing

RESISTANCE = THICKNESS ÷ CONDUCYIVITY

U VALUE = 1 ÷ TOTAL RESISTANCE

| Part | Thickness | Resistivity | Conductivity | Resistance |
| --- | --- | --- | --- | --- |
| ISR | ----- | ----- | ----- | .122 |
| Glass | .005 |  | 1.02 | .0049 |
| ESR | ----- | ----- | ----- | .08 |
|  |  |  | Total Resistance | .2069 |

U value = 1 ÷ .2069 = 4.83 W/m²/ °C

Step 3 Draw data table for Double Glazing

| Part | Thickness | Resistivity | Conductivity | Resistance |
| --- | --- | --- | --- | --- |
| ISR | ----- | ----- | ----- | 1.22 |
| GLASS | .004 | ----- | 1.02 | .00392 |
| SPACE | ----- | ----- | ----- | .170 |
| GLASS | .004 | ----- | 1.02 | .00392 |
| ESR | ----- | ----- | ----- | .08 |
|  |  |  | Total Resistance | .3798 |

U VALUE = 1 ÷ .3798 = 2.63 W/m²/ °C

ANSWER

U VALUE FOR SINGLE GLAZING =4.83 W/m²/ °C

U VALUE FOR DOUBLE GLAZING = 2.63 W/m²/ °C

PART B

SINGLE GLAZING

- Heat Loss in KJ = U value x Area x Temp Difference ÷ 1000

= 4.83        X 25               X 13              ÷ 1000

=1.569 KJ/sec

- Time 1 Litre of oil will last = (Calorific Value of Oil ÷ Heat Loss) ÷ 3600

= (37350 ÷ 1.569) ÷3600

= 6.612

- Amount of oil used = Time the heating is on for ÷ Time 1 Litre will last for

= 3080 ÷ 6.612

= 465.81

- Cost of Oil = Amount of oil in litres used x price of oil per litre

= 465.81 x .80

= €372.65

Answer

Cost of Heat Loss per annum for single glazing = €372.65

DOUBLE GLAZING

- Heat Loss in KJ = U value x Area x Temp Difference ÷ 1000

= 2.73       X 25               X 13              ÷ 1000

=.85475 KJ/sec

- Time 1 Litre of oil will last = (Calorific Value of Oil ÷ Heat Loss) ÷ 3600

= (37350 ÷ .85475) ÷3600

= 12.13

- Amount of oil used = Time the heating is on for ÷ Time 1 Litre will last for

= 3080 ÷ 12.13

= 253.91

- Cost of Oil = Amount of oil in litres used x price of oil per litre

= 153.91 x .80

= €203.12

Answer

Cost of Heat Loss per annum for Double glazing = €203.12

LOW-E DOUBLE GLAZING

- Heat Loss in KJ = U value x Area x Temp Difference ÷ 1000

= 1.1        X 25               X 13              ÷ 1000

=.3575 KJ/sec

- Time 1 Litre of oil will last = (Calorific Value of Oil ÷ Heat Loss) ÷ 3600

= (37350 ÷ .3575) ÷3600

= 29.02

- Amount of oil used = Time the heating is on for ÷ Time 1 Litre will last for

= 3080 ÷ 29.02

= 106.13

- Cost of Oil = Amount of oil in litres used x price of oil per litre

= 106.13 x .80

= €84.90

Answer

Cost of Heat Loss per annum for low-e double glazing = €84.90

Part C

I would recommend for home owners to install Low-e double glazing because.

- Financial Savings

The homeowner will use less oil per annum to heat the house, from the calculation above low-e double glazing costs €84.90 compared to €203 for double glazing and €375 for single glazing.

2) Environmental Friendly

The low-e double glazing reduces the amount of      carbon emissions that are a bye product of burning oil; these emissions are harmful to our planet.

Marking Scheme